EXPERIMENT NO. 16

AIM

To write and simulate an Embedded C program using serial interrupt in 8051 microcontroller such that while a message is continuously transmitted, any key pressed from the keyboard generates an interrupt and is displayed through UART.

ALGORITHM

Configure UART in Mode 1 and Timer 1 for 9600 baud.

Enable serial interrupts.

Continuously transmit “Hello World”.

When a character is received, the serial ISR reads SBUF.

Display the received character through UART.

PROGRAM

#include <reg51.h> #include <stdio.h> void serial_ISR(void) interrupt 4 { char ch; if(RI) { RI = 0; ch = SBUF; printf("\n>>> INTERRUPT RECEIVED: "); printf("%c\n", ch); } } void delay(void) { unsigned int i; for(i = 0; i < 50000; i++); } void main(void) { SCON = 0x50; /* UART mode 1, 8-bit, REN enabled */ TMOD = 0x20; /* Timer 1 mode 2 */ TH1 = 0xFD; /* Baud rate 9600 */ TR1 = 1; IE = 0x90; /* EA = 1, ES = 1 */ TI = 1; while(1) { printf("Hello World\n"); delay(); } }

OUTPUT

RESULT

Thus, the serial interrupt using UART was successfully implemented and verified in Keil simulator.