EXPERIMENT NO. 14

AIM

To write an Embedded C program to display "HELLO WORLD" message using Keil software.

ALGORITHM

Configure UART in Mode 1.

Configure Timer 1 in Mode 2 and load the value for 9600 baud.

Start Timer 1 and enable transmission.

Continuously transmit the message “Hello World!” through UART.

PROGRAM

#include <reg51.h> #include <stdio.h> void main(void) { SCON = 0x50; /* SCON: mode 1, 8-bit UART, enable rcv */ TMOD = 0x20; /* Timer 1 mode 2, 8-bit reload */ TH1 = 0xFD; /* TH1: reload value for 9600 baudrate */ TR1 = 1; /* TR1: timer 1 run */ TI = 1; /* TI: set TI to send first char of UART */ while(1) { printf("Hello World! \n"); } }

OUTPUT

RESULT

Thus the program has been successfully verified and executed.