EXPERIMENT 4
7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

AIM: To write an assembly language program for 7 Segment Display using AT89C51 using Keil and Proteus.

SOFTWARE REQUIRED: Proteus 8 software

ALGORITHM

Send the segment code for digit 0 to Port 2.

Delay and send the codes for digits 1 through 9.

Use a nested delay routine to keep each digit visible.

Repeat continuously.

PROGRAM

ORG 000H
UP: MOV P2,#0C0H
    ACALL DELAY
    MOV P2,#0F9H
    ACALL DELAY
    MOV P2,#0A4H
    ACALL DELAY
    MOV P2,#0B0H
    ACALL DELAY
    MOV P2,#099H
    ACALL DELAY
    MOV P2,#092H
    ACALL DELAY
    MOV P2,#082H
    ACALL DELAY
    MOV P2,#0F8H
    ACALL DELAY
    MOV P2,#080H
    ACALL DELAY
    MOV P2,#090H
    ACALL DELAY

DELAY: MOV R5,#10
H1:    MOV R4,#180
H2:    MOV R3,#255
H3:    DJNZ R3,H3
       DJNZ R4,H2
       DJNZ R5,H1
       RET
END

OUTPUT IMAGE

RESULT

Thus the program has been successfully verified and executed.