EXPERIMENT 5
ARDUINO BASED MANUAL ELECTRONIC COUNTER USING PROTEUS

AIM: To write an Embedded C program for Manual Electronic counter using Arduino IDE and Proteus.

SOFTWARE REQUIRED: Proteus 8 software; Arduino IDE

ALGORITHM

Configure the pushbutton as INPUT_PULLUP and segment pins as outputs.

Read the pushbutton state.

On each button press, move to the next digit state.

Display digits 0 to 9 using the corresponding segment patterns.

After 9, return to 0 and continue.

PROGRAM

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9;
int delay_time=200;

void setup() {
  pinMode(12, INPUT_PULLUP);
  pinMode(1, OUTPUT);
  pinMode(2, OUTPUT);
  pinMode(3, OUTPUT);
  pinMode(4, OUTPUT);
  pinMode(5, OUTPUT);
  pinMode(6, OUTPUT);
  pinMode(7, OUTPUT);
}

void loop() {
  int sensorVal = digitalRead(12);

  if (sensorVal == LOW) {
    x0=true;
    while(x0){
      zero();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x1=true; x0=false; }
    }
    while(x1){
      one();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x2=true; x1=false; }
    }
    while(x2){
      two();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x3=true; x2=false; }
    }
    while(x3){
      three();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x4=true; x3=false; }
    }
    while(x4){
      four();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x5=true; x4=false; }
    }
    while(x5){
      five();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x6=true; x5=false; }
    }
    while(x6){
      six();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x7=true; x6=false; }
    }
    while(x7){
      seven();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x8=true; x7=false; }
    }
    while(x8){
      eight();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x9=true; x8=false; }
    }
    while(x9){
      nine();
      sensorVal=digitalRead(12);
      if(sensorVal==LOW){ x0=true; x9=false; }
    }
  }
}

void zero(){
  for(int i=1;i<7;i++){
    digitalWrite(i,HIGH);
    digitalWrite(7,LOW);
  }
  delay(delay_time);
}

void one(){
  digitalWrite(1,LOW); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,LOW);
  digitalWrite(7,LOW); delay(delay_time);
}

void two(){
  digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,LOW);
  digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,LOW);
  digitalWrite(7,HIGH); delay(delay_time);
}

void three(){
  digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,LOW);
  digitalWrite(7,HIGH); delay(delay_time);
}

void four(){
  digitalWrite(1,LOW); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,HIGH);
  digitalWrite(7,HIGH); delay(delay_time);
}

void five(){
  digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH);
  digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,HIGH);
  digitalWrite(7,HIGH); delay(delay_time);
}

void six(){
  digitalWrite(1,HIGH); digitalWrite(2,LOW); digitalWrite(3,HIGH);
  digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH);
  digitalWrite(7,HIGH); delay(delay_time);
}

void seven(){
  digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,LOW); digitalWrite(5,LOW); digitalWrite(6,LOW);
  digitalWrite(7,LOW); delay(delay_time);
}

void eight(){
  digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,HIGH); digitalWrite(5,HIGH); digitalWrite(6,HIGH);
  digitalWrite(7,HIGH); delay(delay_time);
}

void nine(){
  digitalWrite(1,HIGH); digitalWrite(2,HIGH); digitalWrite(3,HIGH);
  digitalWrite(4,HIGH); digitalWrite(5,LOW); digitalWrite(6,HIGH);
  digitalWrite(7,HIGH); delay(delay_time);
}

OUTPUT IMAGE

RESULT

Thus the program has been successfully verified and executed.